SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2-DEBUGGING & OPTIMISATION TASK

| Course Code: | CSA1223 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL3DEBUGGING & OPTIMISATION TASK |
| Weightage: | 20% | Total Marks: | 100 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: | YASHWANTH.G | Reg.No.: | 192524275 |
| Department: | AIDS | Date: |  |

ANNEXURE A

- A program performing signed integer addition using 2’s complement produces incorrect results when adding large values (e.g., +120 and +50 in 8-bit representation). Debug the issue by analyzing the binary computation and identifying whether overflow has occurred. Propose a solution to correctly detect and handle overflow in such cases.

- A processor using a ripple carry adder shows significant delay in executing arithmetic operations in a real-time system. Debug the performance bottleneck by examining how carry propagates through each bit. Suggest an optimized solution using a carry lookahead adder, and explain how it reduces delay.

- A multiplication unit implementing Booth’s algorithm gives incorrect results for certain negative operands. Debug the step-by-step execution of the algorithm, focusing on bitpair checking, arithmetic shifts, and sign extension. Identify the possible source of error and suggest corrections to ensure accurate signed multiplication.

- An embedded system using the restoring division algorithm experiences inefficiency due to repeated restoration steps, leading to increased execution time. Debug the algorithm’s workflow and identify where unnecessary operations occur. Propose an optimized approach using the non-restoring division method, explaining how it improves performance.

- A scientific application using IEEE 754 single precision floating-point representation produces inaccurate results when adding very small and very large numbers. Debug the issue by analyzing exponent alignment, normalization, and rounding errors. Suggest optimization techniques, such as using double precision or algorithmic adjustments, to improve accuracy.

Code of Conduct:

I G.Yashwanth(Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:YASHWANTH

MARKS ALLOCATION

|  | Identification of Errors / Bugs(20%) | Debugging Approach & Analysis(20%) | Correctnes s of Debugged Solution(2 5%) | Optimization Techniques Applied(20%) | Testing and Documentation(15 %) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Identification of Errors / Bugs | 20% | Accurately identifies all errors and issues with complete understanding of the problem | Identifies most errors with minor omissions | Identifies limited errors; misses key issues | Unable to identify errors or misunderstands the problem |
| Debugging Approach & Analysis | 20% | Uses effective debugging techniques with clear analysis and root cause identification | Applies suitable debugging methods with minor gaps | Uses basic debugging methods with limited analysis | No systematic debugging approach followed |
| Correctness of Debugged Solution | 25% | Provides completely corrected solution with accurate functionality and expected output | Provides working solution with minor errors | Partially correct solution with limited functionality | Incorrect solution or fails to resolve errors |
| Optimization Techniques Applied | 20% | Applies efficient optimization methods to improve performance, memory usage and quality | Performs optimization with noticeable improvements | Limited optimization with minimal improvements | No optimization applied or inefficient implementation |
| Testing and Documentation | 15% | Performs complete testing with proper validation and clear documentation | Provides adequate testing and documentation with minor issues | Limited testing and incomplete documentation | No proper testing or documentation provided |

Q1. Debugging 2's Complement Overflow Error Identification:

- Adding +120 and +50 in 8-bit 2's complement exceeds the representable range (-128 to +127).

- Binary:

  - +120 = 01111000  o 	+50 = 00110010  o 	Sum = 10101010 (incorrect because overflow occurred).

Debugging Analysis:

- Overflow occurs when two positive numbers produce a negative result.   	Carry into the sign bit is different from carry out of the sign bit.

Correct Debugged Solution:

- Detect overflow using:  o Overflow = Carry into MSB  Carry out of MSB   If overflow is detected:

  - Display an overflow message.

  - Use a larger data type (16-bit/32-bit) for the calculation.

Optimization:

- Implement a hardware overflow flag.

- Perform automatic range checking before arithmetic operations.  Testing & Documentation:

- Test:

  - 120 + 50 → Overflow detected.

  - 30 + 20 → 50 (Correct)  o 	-70 + -80 → Overflow detected.

- Document the overflow detection logic and expected outputs.

Q2. Ripple Carry Adder Performance Error Identification:

- Ripple Carry Adder has high delay because each carry must propagate through every bit sequentially.

Debugging Analysis:

- Total delay increases with the number of bits.

- In real-time systems this slows arithmetic execution.

Correct Debugged Solution:

- Replace Ripple Carry Adder with Carry Look-Ahead Adder (CLA).

- CLA computes carry signals in parallel using Generate (G) and Propagate (P).

Optimization:

- Parallel carry generation reduces propagation delay.

- Improves processor speed and overall throughput.

Testing & Documentation:

- Compare RCA and CLA for:  o 	8-bit addition  o 	16-bit addition  o 	32-bit addition

- Record execution time and verify identical outputs.

Q3. Booth's Multiplication Algorithm Error Identification:

- Incorrect multiplication occurs due to:

  - Wrong bit-pair checking.

  - Improper arithmetic right shift.

  - Incorrect sign extension.

Debugging Analysis:

- Verify every Booth iteration:

  - Check Q₀ and Q₋₁.

  - Perform Add/Subtract correctly.

  - Apply arithmetic shift while preserving the sign bit.

Correct Debugged Solution:

- Correct sign extension after every arithmetic shift.   	Ensure proper updates of accumulator (A), multiplier (Q), and Q₋₁.  Optimization:

- Skip unnecessary additions for consecutive 0s or 1s.

- Improves multiplication efficiency.

Testing & Documentation:

- Test:

  - 5 × (-3)  o 	(-7) × (-2)  o 	(-8) × 6

- Verify results with manual calculations.

Q4. Restoring vs Non-Restoring Division Error Identification:

- Restoring Division repeatedly restores the previous remainder after unsuccessful subtraction.   This increases execution time.

Debugging Analysis:

- Restoration step is the main performance bottleneck.

Correct Debugged Solution:

- Replace with Non-Restoring Division.

- Instead of restoring immediately:

  - Perform addition or subtraction in the next cycle based on the sign of the remainder.

Optimization:

- Eliminates unnecessary restore operations.

- Reduces clock cycles.

- Improves processor efficiency.

Testing & Documentation:

- Test:

  - 25 ÷ 5  o 	30 ÷ 6  o 	15 ÷ 4

- Compare execution time with Restoring Division and verify quotient and remainder.

Q5. IEEE 754 Floating Point Addition Error Identification:

- Inaccurate results occur when adding very small and very large floating-point numbers.

- Causes:

  - Exponent alignment.

  - Normalization.  o 	Rounding errors.

Debugging Analysis:

- During exponent alignment, smaller numbers lose significant bits.

- Precision decreases due to rounding.

Correct Debugged Solution:

- Use IEEE 754 Double Precision (64-bit).

- Normalize correctly after addition.

- Apply proper rounding mode.

Optimization:

- Use compensated summation (e.g., Kahan Summation).

- Perform calculations in higher precision to reduce numerical errors.  Testing & Documentation:

- Test:

  - 1e20 + 1  o 	1.5 + 2.75  o 	0.000001 + 1000000

- Compare single-precision and double-precision outputs and document accuracy improvements.