## 1. Real-Time Embedded System I/O Communication

### Given Constraints

- CPU utilization must be below 15%

- Multiple sensors generate interrupts every 1 ms

- Use vectored interrupt handling

- Select the I/O method with minimum latency

### Design

Use memory-mapped I/O + vectored interrupt controller + DMA.

### Components

- Sensors

- Interrupt Controller

- CPU

- DMA Controller

- Main Memory

- Memory-mapped I/O registers

### Diagram

Sensors

↓

Interrupt Controller

↓

Vectored Interrupt

↓

CPU

↓

DMA

↓

Main Memory

### Working

- Sensors continuously generate data.

- When a sensor needs attention, it generates an interrupt.

- The interrupt controller identifies the sensor and sends its vector number to the CPU.

- CPU directly executes the corresponding ISR.

- DMA transfers sensor data directly to memory.

- CPU does not handle every data byte.

### Why Memory-Mapped I/O?

Memory-mapped I/O provides fast access to device registers and reduces the time required for I/O operations.

### Final Choice

Memory-mapped I/O + vectored interrupts + DMA

Reason: Low latency and reduced CPU utilization help keep CPU usage below 15%.

# 2. High-Speed Storage I/O Subsystem

### Given Constraints

- Throughput must be greater than 3 GB/s

- Must use NVMe over PCIe

- CPU should not handle bulk data

- DMA and interrupt notification are required

### Design

Use an NVMe SSD connected through PCIe with DMA.

### Components

- CPU

- PCIe Bus

- NVMe Controller

- NVMe SSD

- DMA Controller

- Main Memory

- Interrupt Controller

### Diagram

CPU

│

NVMe Command

│

▼

PCIe Bus

│

▼

NVMe Controller

│

▼

NVMe SSD

│

DMA

│

▼

Main Memory

│

▼

Interrupt → CPU

### Working

- CPU sends an NVMe command.

- PCIe transfers the command to the NVMe controller.

- NVMe SSD reads/writes the data.

- DMA transfers bulk data directly between SSD and main memory.

- CPU receives an interrupt after the transfer is completed.

### Why DMA?

DMA transfers large amounts of data without continuous CPU involvement.

### Final Choice

NVMe + PCIe + DMA + Interrupt

Reason: Provides high-speed storage transfer above 3 GB/s while reducing CPU workload.

# 3. Multi-Device Communication Architecture

### Given Constraints

- 6 USB peripherals

- 2 Thunderbolt high-speed devices

- Avoid interrupt starvation

- Use hardware and software interrupts

- Fair bus arbitration

### Design

Use separate USB and Thunderbolt controllers connected through a fair bus arbiter.

### Diagram

CPU

↓

Bus Arbiter

↙       ↘

USB Controller   Thunderbolt Controller

↓ ↓ ↓ ↓ ↓ ↓                       ↓     ↓

U1 U2 U3 U4 U5 U6     T1    T2

### Working

- Six devices communicate through the USB controller.

- Two high-speed devices use the Thunderbolt controller.

- The bus arbiter controls which device gets access to the bus.

- Round-robin arbitration gives each device a fair opportunity.

- Hardware interrupts handle urgent device events.

- Software interrupts handle background tasks.

### How is Starvation Avoided?

Use round-robin arbitration, where each device gets its turn.

### Final Choice

USB Controller + Thunderbolt Controller + Round-Robin Arbiter + Hardware/Software Interrupts

# 4. Data Acquisition System

### Given Constraints

- Continuous data rate = 500 MB/s

- Minimum CPU intervention

- Compare DMA and interrupt-driven I/O

- Use memory-mapped I/O

### Design

Use memory-mapped I/O + DMA + ring buffer.

### Diagram

Data Acquisition Device

│

500 MB/s

│

▼

DMA Controller

│

▼

Ring Buffer

Main Memory

│

▼

CPU

▲

│

DMA Completion

Interrupt

### Working

- The data acquisition device continuously generates data.

- Device registers are accessed using memory-mapped I/O.

- DMA transfers data directly into the ring buffer.

- CPU processes the data from memory.

- DMA generates an interrupt after completing a block transfer.

### DMA vs Interrupt-Driven I/O

| Factor | DMA | Interrupt-Driven I/O |
| --- | --- | --- |
| CPU involvement | Very low | High |
| Data transfer | Device → Memory | Device → CPU → Memory |
| CPU overhead | Low | High |
| Suitable for 500 MB/s | Yes | No |
| Performance | High | Lower |
| Continuous transfer | Suitable | Less suitable |

### Final Choice

DMA is preferred.

Reason: 500 MB/s is a very high continuous data rate. DMA transfers data directly to memory and greatly reduces CPU intervention.

# 5. Nested Interrupt Handling Mechanism

### Given Constraints

- Support nested interrupts

- Interrupt latency must be less than 10 µs

- High-priority devices use vectored interrupts

- Background tasks use software interrupts

### Design

Use a priority-based interrupt controller with nested interrupts.

### Diagram

Devices

│

▼

Priority Interrupt

Controller

┌───────┴────────┐

│                                             │

▼                                            ▼

High Priority                      Low Priority

Device                                 Device

│                                          │

▼                                        ▼

Vectored ISR                      Normal ISR

│                 │

└──────┬─────────┘

▼

Nested Interrupt

│

▼

Restore Context

│

▼

Continue ISR

Background Tasks

│

▼

Software Interrupt

### Working

- Devices generate interrupts.

- The interrupt controller assigns priority.

- High-priority devices use vectored interrupts.

- If a high-priority interrupt occurs while a low-priority ISR is running, it temporarily interrupts the low-priority ISR.

- CPU saves the current context.

- High-priority ISR executes first.

- CPU restores the previous context and continues the interrupted ISR.

- Background tasks use software interrupts.

### How to Achieve <10 µs Latency?

- Use vectored interrupts.

- Keep ISRs short.

- Use hardware priority levels.

- Avoid unnecessary processing inside the ISR.

- Use fast interrupt-controller hardware.

### Final Choice

Priority-based nested vectored interrupts + software interrupts

Reason: High-priority devices receive immediate service while background tasks are handled separately.

窗体顶端

窗体底端