1. DMA-Based Data Transfer Algorithm

Purpose: Transfer data from an I/O device to memory with minimum CPU involvement.

BEGIN

Initialize DMA Controller

Initialize I/O Device

Set Source = I/O Device

Set Destination = Main Memory

Set Transfer Size

Enable DMA

Start DMA Transfer

WHILE transfer is not complete

CPU performs other tasks

END WHILE

DMA generates interrupt

IF interrupt received THEN

Check transfer status

IF transfer successful THEN

Display "Transfer Completed"

ELSE

Display "Transfer Error"

END IF

END IF

Disable DMA

END

Key point: DMA transfers data directly to memory, so CPU involvement is low.

## 2. Vectored Interrupt Handling

Purpose: Give priority to interrupts from multiple devices and execute the correct ISR.

BEGIN

Initialize Interrupt Controller

Set Priority for each device

Enable Interrupts

WHILE system is running

IF interrupt occurs THEN

Identify interrupt source

Select highest-priority interrupt

Get interrupt vector

Call corresponding ISR

Execute ISR

Clear interrupt flag

Return to previous task

END IF

END WHILE

END

### Simple flow

Devices

↓

Interrupt Controller

↓

Check Priority

↓

Get Vector

↓

Execute ISR

↓

Return to Task

## 3. Dynamic I/O Selection

Purpose: Select memory-mapped I/O or I/O-mapped I/O according to system requirements.

BEGIN

Read latency requirement

Read bandwidth requirement

IF latency requirement is HIGH THEN

Select Memory-Mapped I/O

ELSE IF bandwidth requirement is HIGH THEN

Select Memory-Mapped I/O

ELSE

Select I/O-Mapped I/O

END IF

Configure selected I/O method

Start I/O operation

END

### Simple decision

I/O Requirement

↓

Check Latency

↓

High? ── Yes ──→ Memory-Mapped I/O

↓ No

↓

Check Bandwidth

↓

High? ── Yes ──→ Memory-Mapped I/O

↓ No

↓

I/O-Mapped I/O

Key point: Memory-mapped I/O is preferred when fast access and high performance are required.

## 4. Bus Arbitration Algorithm

Purpose: Manage multiple devices using PCIe, USB, and Thunderbolt fairly.

BEGIN

Initialize PCIe devices

Initialize USB devices

Initialize Thunderbolt devices

Create device queue

WHILE devices need bus access

Check all requesting devices

IF high-priority request exists THEN

Grant bus to high-priority device

ELSE

Select next device using Round-Robin

END IF

Allow device to use bus

Complete data transfer

Release bus

Move to next device

END WHILE

END

### Simple flow

Devices

↓

Check Requests

↓

Priority Check

↓

Round-Robin Selection

↓

Grant Bus

↓

Data Transfer

↓

Release Bus

Key point: Round-robin arbitration provides fair access and prevents starvation.

## 5. Hybrid I/O Communication

Purpose: Use polling for low-speed devices and DMA with interrupts for high-speed devices.

BEGIN

Initialize all devices

FOR each device

Check device speed

IF device is LOW-SPEED THEN

Use Polling

WHILE data is available

Read data

END WHILE

ELSE

Use DMA

Start DMA transfer

WAIT for DMA interrupt

IF interrupt received THEN

Check transfer status

Process data

END IF

END IF

END FOR

END

### Simple flow

Device

↓

Check Speed

↙       ↘

Low Speed   High Speed

↓           ↓

Polling       DMA

↓

Interrupt

↓

Process Data