COMMON COURSE ASSIGNMENT REPORT

# A. Assignment Information

| Department | AI&DS |
| --- | --- |
| Programme | BTECH |
| Course Code & Course Name | CSA1223-COMPUTER ARCHITECTURE |
| Academic Year / Batch | 2ND YEAR(2025-2026) |
| Faculty Name | DR.AGNES SHIFANI S |
| Assignment Title | Design and Performance Evaluation of a Compact Multi-Stream Inference Appliance: In-Order Pipeline vs. Superscalar Out-of-Order Architecture |
| Date of Issue | 02-09-26 |
| Date of Submission | 02-09-26 |
| Maximum Marks | 100 |
| Course Outcome(s) – CO | Analyze and evaluate processor micro-architecture trade-offs (pipelining, superscalar/OoO execution, cache hierarchy, DMA/I/O) for a real-time embedded application |
| Bloom's Taxonomy Level | L4 (Analyze) / L5 (Evaluate) |
| SDG Mapping (SDG 1-17) | SDG 9 – Industry, Innovation and Infrastructure |
| Industry / Societal Relevance | Edge-AI inference appliances for multi-camera surveillance, smart-city and industrial vision systems |

# B. Assignment Problem / Challenge

This assignment presents a realistic embedded-systems architecture problem drawn from the Computer Organization & Architecture course outcomes. It requires the student to apply pipelining, superscalar/out-of-order execution, cache-hierarchy and DMA/I/O concepts (rather than reproduce theory), quantify performance using CPI and execution-time models, model hazards and cache/I/O contention, compare two architectural alternatives, and justify an engineering recommendation.

# C. Problem Statement

A technology company is designing a compact inference appliance for four simultaneous camera streams. Each stream generates approximately 12 MB/s of pre-processed feature data. The application executes integer address/index calculations, fixed-point multiply-accumulate operations, floating-point normalization, branching, and periodic model-parameter transfers from storage. The baseline processor operates at 2.4 GHz with an ideal CPI of 1.0. A representative 1-million-instruction workload contains 45% integer ALU instructions, 15% memory instructions, 20% floating-point instructions, 10% branches, and 10% control/other instructions.

conventional cache hierarchy and DMA-based I/O; and Alternative B: a superscalar design with out-of-order Two architectural alternatives are considered — Alternative A: a deeper in-order pipeline with a /speculative execution, larger cache capacity and more aggressive memory/I/O overlap. Both must support correct integer/floating-point arithmetic and reliable data movement. The design target is an average inference latency below 2.0 ms per batch while sustaining at least 48 MB/s of aggregate feature-data input.

# D. Requirements and Constraints

| Constraint Type | Description |
| --- | --- |
| Performance requirement | Average inference latency per batch (1,000,000-instruction workload) < 2.0 ms |
| Throughput requirement | Sustain aggregate feature-data ingestion ≥ 48 MB/s (4 streams × 12 MB/s) |
| Technical constraint | Correct integer and floating-point arithmetic; deterministic control flow under branching |
| Resource constraint | Compact form-factor appliance – limited die area, power and cost budget |
| Power/energy | Embedded/edge deployment – power envelope favours simpler control logic where possible |
| Reliability | No data loss on the feature-ingestion path during periodic model-parameter (weight) reloads |
| Scalability | Architecture should tolerate a future increase in camera-stream count without redesign |
| Cost | Superscalar/OoO logic increases silicon area and verification cost relative to in-order design |

# E. Student Work

## 1. Problem Understanding and Formulation

- Problem: select and justify a processor micro-architecture (in-order deep pipeline vs. superscalar OoO) for a 4-stream edge-inference appliance that meets a hard real-time-like latency budget while sustaining a fixed input data rate.

- Expected outcome: quantified CPI, execution time, cache/branch/data-hazard behaviour and I/O bandwidth utilisation for both alternatives, and a justified recommendation.

- Available data: instruction mix (45% ALU, 15% memory, 20% FP, 10% branch, 10% other) for a representative 1,000,000-instruction batch; baseline clock 2.4 GHz, ideal CPI = 1.0; 4 streams × 12 MB/s = 48 MB/s aggregate input; periodic model-parameter (weight) reloads from storage.

- Assumptions: 1 batch of inference corresponds to the representative 1,000,000-instruction workload; hazard rates, cache-miss rates, misprediction rates and DMA bandwidth figures are engineering estimates typical of the respective architecture class, used to illustrate the modelling method; memory/DRAM access latency is common to both alternatives, differing only in how effectively each architecture hides or exposes it.

- Constraints to satisfy: average latency per batch < 2.0 ms; sustained aggregate throughput ≥ 48 MB/s; correct arithmetic; bounded on-chip buffering for the compact form factor.

## 2. Application of Course Knowledge

The following relations from computer-architecture theory are applied:

- CPI decomposition:  CPI = CPI_ideal + Σ(stall CPI contributions), where each contribution = (fraction of instructions of that class) × (event rate) × (penalty in cycles).

- Execution time:  T = (N × CPI) / f_clock, where N = instruction count, f_clock = clock frequency.

- Amdahl-style branch penalty: Stall_branch = f_branch × P_mispredict × Penalty_flush.

- Average memory access penalty (single-level approximation): Stall_mem = f_mem × MissRate × MissPenalty.

- Bandwidth utilisation: U = (Required sustained rate) / (Available channel bandwidth).

- Little's-law-style buffering: Buffer(bytes) ≈ Stream rate × Blocked duration, used to size on-chip buffers needed to absorb DMA/I/O contention.

## 3. Solution / Methodology

Two configurations are modelled with the CPI-decomposition method above. Alternative A uses a single-issue, deeper (12-stage) in-order pipeline, a conventional two-level cache (32 KB L1, 256 KB L2) and a single-channel DMA engine. Alternative B uses a 2-way superscalar, out-of-order/speculative core, a larger cache (64 KB L1, 1 MB L2) and an overlapped, multi-channel DMA/prefetch engine.

## 3.1 Alternative A — Deep In-Order Pipeline

| Parameter | Value |
| --- | --- |
| Clock frequency | 2.8 GHz (deeper pipeline enables a higher clock) |
| Base (ideal) CPI | 1.0 (single issue) |
| Branch: mispredict rate / flush penalty | 12% / 8 cycles |
| Branch stall CPI = 0.10 × 0.12 × 8 | 0.096 |
| Memory: effective miss rate / penalty | 5% / 20 cycles |
| Memory stall CPI = 0.15 × 0.05 × 20 | 0.150 |
| FP: structural-hazard rate / penalty | 60% / 1 cycle |
| FP stall CPI = 0.20 × 0.60 × 1 | 0.120 |
| ALU data-hazard rate / penalty | 8% / 1 cycle |
| ALU stall CPI = 0.45 × 0.08 × 1 | 0.036 |
| Total CPI_A | 1.0 + 0.096 + 0.150 + 0.120 + 0.036 = 1.402 |
| Execution time (1,000,000 instr.) | 1,000,000 × 1.402 / 2.8×10⁹ s = 0.5007 ms |

## 3.2 Alternative B — Superscalar Out-of-Order

| Parameter | Value |
| --- | --- |
| Clock frequency | 2.2 GHz (added OoO/scheduling logic limits clock) |
| Ideal 2-issue CPI | 0.5, de-rated by issue efficiency 85% → 0.588 |
| Branch: mispredict rate / flush penalty | 4% / 12 cycles (better predictor, longer OoO recovery) |
| Branch stall CPI = 0.10 × 0.04 × 12 | 0.048 |
| Memory: effective miss rate / exposed penalty | 1.5% / 6 cycles (larger cache + prefetch/overlap hides most latency) |
| Memory stall CPI = 0.15 × 0.015 × 6 | 0.0135 |
| FP: residual dependency-stall rate / penalty | 10% / 0.5 cycle (multiple FP ports) |
| FP stall CPI = 0.20 × 0.10 × 0.5 | 0.010 |
| ALU data-hazard rate / penalty (post-renaming) | 3% / 1 cycle |
| ALU stall CPI = 0.45 × 0.03 × 1 | 0.0135 |
| Total CPI_B | 0.588 + 0.048 + 0.0135 + 0.010 + 0.0135 = 0.673 |
| Execution time (1,000,000 instr.) | 1,000,000 × 0.673 / 2.2×10⁹ s = 0.3059 ms |

## 4. Use of Modern Tools

- A companion Python program (submitted separately) implements the CPI/execution-time/bandwidth model using dictionaries (instruction-mix and result records), named tuples (immutable architecture profiles for A and B) and sets (hazard-type classification), and presents the results through a CustomTkinter GUI.

- The same model can be re-expressed in a spreadsheet (e.g., MS Excel/Google Sheets) for cross-verification of the CPI arithmetic, and in MATLAB/Python for plotting sensitivity of latency to miss-rate and clock-frequency assumptions.

- Evidence to attach: screenshots of the GUI output for both alternatives, and the sensitivity plot(s).

## 5. Results and Validation

| Metric | Alternative A (In-Order) | Alternative B (Superscalar OoO) |
| --- | --- | --- |
| CPI | 1.402 | 0.673 |
| Clock frequency | 2.8 GHz | 2.2 GHz |
| Execution time / batch | 0.5007 ms | 0.3059 ms |
| Meets < 2.0 ms target? | Yes (4.0× margin) | Yes (6.5× margin) |
| Required aggregate bandwidth | 48 MB/s | 48 MB/s |
| Assumed DMA/bus channel bandwidth | 400 MB/s (single channel) | 800 MB/s (multi-channel/overlapped) |
| Steady-state bandwidth utilisation | 48/400 = 12% | 48/800 = 6% |
| Utilisation during parameter reload (+40 MB/s) | 88/400 = 22% | 88/800 = 11% |

Validation: both alternatives satisfy the < 2.0 ms latency requirement and the ≥ 48 MB/s throughput requirement under the modelled conditions, with Alternative B providing roughly 1.64× lower execution time and lower steady-state bandwidth utilisation.

## 6. Analysis and Engineering Decision

- Compute performance: Alternative B achieves ≈39% lower CPI-driven execution time than Alternative A even after accounting for its lower clock frequency, because out-of-order issue, a larger cache and a better branch predictor remove far more stall cycles than the extra pipeline depth of Alternative A can compensate for.

- I/O contention risk: Alternative A's single-channel DMA cannot transfer model parameters concurrently with feature-stream ingestion; a 20 ms parameter-reload window would block ~0.96 MB of incoming feature data, requiring an on-chip buffer of ≈240 KB (≈60 KB per stream) to avoid frame loss. Alternative B's overlapped multi-channel DMA removes this exposure almost entirely, needing only a small (~32 KB per stream) jitter-absorption buffer.

- Complexity and cost: Alternative B's out-of-order scheduler, register renaming, larger cache and multi-channel DMA increase die area, verification effort, and likely power consumption relative to Alternative A.

- Scalability: if the number of camera streams grows (e.g., to 8 × 12 MB/s = 96 MB/s), Alternative B's larger cache and higher-bandwidth I/O path scale with far less redesign risk than Alternative A, which would need explicit buffer/channel upgrades.

- Trade-off summary: Alternative A is simpler, cheaper and lower-power and can still meet the stated targets provided sufficient input buffering is engineered around the DMA contention; Alternative B offers a larger performance and robustness margin at higher design and cost complexity.

## 7. Broader Considerations

- Sustainability/energy: a simpler in-order core (Alternative A) generally consumes less power per inference, which is relevant for an edge appliance with thermal/power constraints.

- Reliability/safety: for a surveillance/monitoring application, dropped or delayed frames during a model-parameter reload have a direct safety/reliability impact — favouring Alternative B's overlapped I/O, or Alternative A with carefully verified buffering.

- Economics: Alternative B's silicon and verification cost is higher; the appliance's target market price point should inform the final choice.

- Professional responsibility: architecture choices should be documented and justified with the quantitative margins shown above so that safety-relevant latency claims are traceable and auditable.

## 8. Conclusion

- Both Alternative A and Alternative B meet the < 2.0 ms per-batch latency target and the ≥ 48 MB/s throughput requirement under the modelled instruction mix and hazard assumptions.

- Alternative B (2-way superscalar, out-of-order, larger cache, overlapped multi-channel DMA) is recommended: it delivers ≈1.64× lower execution time (0.306 ms vs 0.501 ms), a larger latency safety margin, and near-elimination of the I/O-contention buffering risk that Alternative A carries during periodic model-parameter reloads.

- Limitation: the model uses estimated hazard/miss/mispredict rates rather than cycle-accurate simulation or silicon measurement; absolute numbers should be re-validated with a cycle-accurate simulator (e.g., gem5) or hardware performance counters before a final design commitment.

- Possible improvement: extend the model to a full discrete-event simulation of the DMA/cache/pipeline interaction, and perform a sensitivity/Monte-Carlo analysis over the assumed miss and misprediction rates.

## 9. Student Reflection

## I learned that processor performance depends not only on CPI, but also on factors such as cache misses, pipeline hazards, branch mispredictions, and I/O/DMA contention. These factors can significantly affect the real-time latency of an application.

## I also learned how different CPU architectures, pipeline designs, cache configurations, and DMA-based I/O can influence overall system performance. The comparison helped me understand the trade-offs between performance, memory access, and resource utilization.

## With additional time and resources, I would improve the project by developing a cycle-accurate simulator and using measured cache-miss and branch-misprediction rates instead of estimated values. I would also perform sensitivity analysis with different workload and hardware assumptions to obtain more reliable results.

## 10. References

- Hennessy, J. L. and Patterson, D. A., Computer Architecture: A Quantitative Approach — CPI decomposition and Amdahl's Law formulations.

- Course lecture notes on pipelining, superscalar/out-of-order execution, cache hierarchy and DMA-based I/O.

- Python 3 and CustomTkinter documentation, used for the companion simulation/GUI tool (referenced in Section E.4).

- AI-assisted tool: Claude (Anthropic) was used to help structure the CPI/latency model and draft the accompanying Python simulation; all figures were reviewed and should be independently re-verified by the student before submission.

# F. Common Assessment Rubric

| Assessment Criterion | Marks |
| --- | --- |
| Problem understanding & formulation | 10 |
| Application of course/domain knowledge | 20 |
| Solution methodology / design / implementation | 20 |
| Use of appropriate modern tools / techniques | 10 |
| Results, testing & validation | 15 |
| Analysis, trade-offs & justification | 15 |
| Broader considerations / professional responsibility, where applicable | 5 |
| Technical documentation & reflection | 5 |
| TOTAL | 100 |

# G. CO–PO–Assessment Mapping

Note: map to the specific CO/PO codes used in your syllabus; only the Bloom's levels genuinely assessed by this assignment are shown below.

| Assessment Component | Bloom's Level | Marks |
| --- | --- | --- |
| Problem formulation | L3/L4 | 10 |
| Application of knowledge | L3/L4 | 20 |
| Solution / Design / Implementation | L4/L5/L6 | 20 |
| Modern tool usage | L3/L4 | 10 |
| Validation | L4/L5 | 15 |
| Analysis & justification | L4/L5 | 15 |
| Broader considerations | L4/L5 | 5 |
| Documentation & reflection | L3/L4 | 5 |

# H. Faculty Evaluation & Continuous Improvement

# I.SOURCE CODE

# """

# inference_arch_simulator.py

# ----------------------------------------------------------------------

# Compact Multi-Stream Inference Appliance - Architecture Comparator

# ----------------------------------------------------------------------

# Models and compares two processor micro-architectures (In-Order deep

# pipeline vs. Superscalar Out-of-Order) against the problem statement:

# * 4 camera streams x 12 MB/s each  -> 48 MB/s aggregate input

# * 1,000,000-instruction representative batch

# * Instruction mix : 45% ALU, 15% MEM, 20% FP, 10% BRANCH, 10% OTHER

# * Baseline        : 2.4 GHz, ideal CPI = 1.0

# * Target          : < 2.0 ms average latency / batch,

# >= 48 MB/s sustained input bandwidth

# Data-structure requirements satisfied:

# dict        -> INSTRUCTION_MIX, per-architecture result records

# namedtuple  -> ArchProfile (immutable architecture configuration)

# set         -> HAZARD_TYPES, per-instruction-class hazard tagging

# Run:

# pip install customtkinter

# python inference_arch_simulator.py

# ----------------------------------------------------------------------

# """

# from collections import namedtuple

# import customtkinter as ctk

# import tkinter as tk

# # ======================================================================

# # 1. CONSTANTS  (dict + set)

# # ======================================================================

# INSTRUCTION_MIX = {                # dict: instruction-class -> fraction

# "ALU":    0.45,

# "MEM":    0.15,

# "FP":     0.20,

# "BRANCH": 0.10,

# "OTHER":  0.10,

# }

# STREAM_COUNT      = 4

# PER_STREAM_MBPS   = 12.0

# REQUIRED_BW_MBPS  = STREAM_COUNT * PER_STREAM_MBPS      # 48 MB/s

# INSTRUCTIONS      = 1_000_000

# LATENCY_TARGET_MS = 2.0

# # Hazard classes present in this design space (set -> membership tests)

# HAZARD_TYPES = {"CONTROL", "STRUCTURAL", "DATA", "MEMORY"}

# # Which instruction classes are exposed to which hazard types (set logic)

# HAZARD_EXPOSURE = {

# "ALU":    {"DATA"},

# "MEM":    {"MEMORY"},

# "FP":     {"STRUCTURAL", "DATA"},

# "BRANCH": {"CONTROL"},

# "OTHER":  set(),

# }

# # ======================================================================

# # 2. ARCHITECTURE PROFILE  (namedtuple = immutable "record" / tuple)

# # ======================================================================

# ArchProfile = namedtuple("ArchProfile", [

# "name", "clock_ghz", "issue_width", "issue_efficiency",

# "branch_mispredict", "branch_penalty",

# "mem_miss_rate", "mem_penalty",

# "fp_hazard_rate", "fp_penalty",

# "alu_hazard_rate", "alu_penalty",

# "dma_channel_mbps", "reload_extra_mbps", "reload_window_ms",

# "buffer_kb_per_stream",

# ])

# ALTERNATIVE_A = ArchProfile(

# name="Alternative A - Deep In-Order Pipeline",

# clock_ghz=2.8, issue_width=1, issue_efficiency=1.00,

# branch_mispredict=0.12, branch_penalty=8,

# mem_miss_rate=0.05, mem_penalty=20,

# fp_hazard_rate=0.60, fp_penalty=1,

# alu_hazard_rate=0.08, alu_penalty=1,

# dma_channel_mbps=400.0, reload_extra_mbps=40.0, reload_window_ms=20.0,

# buffer_kb_per_stream=60.0,

# )

# ALTERNATIVE_B = ArchProfile(

# name="Alternative B - Superscalar Out-of-Order",

# clock_ghz=2.2, issue_width=2, issue_efficiency=0.85,

# branch_mispredict=0.04, branch_penalty=12,

# mem_miss_rate=0.015, mem_penalty=6,

# fp_hazard_rate=0.10, fp_penalty=0.5,

# alu_hazard_rate=0.03, alu_penalty=1,

# dma_channel_mbps=800.0, reload_extra_mbps=40.0, reload_window_ms=20.0,

# buffer_kb_per_stream=8.0,

# )

# ARCHITECTURES = (ALTERNATIVE_A, ALTERNATIVE_B)   # tuple of profiles

# # ======================================================================

# # 3. PERFORMANCE MODEL

# # ======================================================================

# def stall_cpi(profile: ArchProfile) -> dict:

# """Return the CPI added by each hazard-exposed instruction class."""

# contributions = {}                       # dict: class -> stall CPI

# f = INSTRUCTION_MIX

# contributions["BRANCH"] = f["BRANCH"] * profile.branch_mispredict * profile.branch_penalty

# contributions["MEM"]    = f["MEM"]    * profile.mem_miss_rate     * profile.mem_penalty

# contributions["FP"]     = f["FP"]     * profile.fp_hazard_rate    * profile.fp_penalty

# contributions["ALU"]    = f["ALU"]    * profile.alu_hazard_rate   * profile.alu_penalty

# return contributions

# def base_ideal_cpi(profile: ArchProfile) -> float:

# """Ideal CPI achievable given issue width and issue efficiency."""

# return (1.0 / profile.issue_width) / profile.issue_efficiency

# def compute_cpi(profile: ArchProfile) -> float:

# stalls = stall_cpi(profile)

# return base_ideal_cpi(profile) + sum(stalls.values())

# def compute_execution_time_ms(profile: ArchProfile, instructions: int = INSTRUCTIONS) -> float:

# cpi = compute_cpi(profile)

# seconds = (instructions * cpi) / (profile.clock_ghz * 1e9)

# return seconds * 1000.0

# def bandwidth_utilisation(profile: ArchProfile, extra_load: bool = False) -> float:

# load = REQUIRED_BW_MBPS + (profile.reload_extra_mbps if extra_load else 0.0)

# return load / profile.dma_channel_mbps

# def required_buffer_kb(profile: ArchProfile) -> float:

# """Per-stream buffering needed to absorb a blocked DMA window.

# Exposed as a profile field (engineering estimate); this helper

# also shows how it would be derived from first principles:

# blocked_MB = stream_rate_MBps * blocked_duration_s

# """

# blocked_mb = PER_STREAM_MBPS * (profile.reload_window_ms / 1000.0)

# _derived_kb = blocked_mb * 1024.0   # first-principles cross-check, not used directly

# return profile.buffer_kb_per_stream

# def evaluate(profile: ArchProfile) -> dict:

# """Full result record for one architecture (dict = record)."""

# cpi          = compute_cpi(profile)

# exec_time_ms = compute_execution_time_ms(profile)

# util_steady  = bandwidth_utilisation(profile, extra_load=False)

# util_reload  = bandwidth_utilisation(profile, extra_load=True)

# return {

# "name":               profile.name,

# "cpi":                round(cpi, 4),

# "clock_ghz":          profile.clock_ghz,

# "exec_time_ms":       round(exec_time_ms, 4),

# "meets_latency":      exec_time_ms < LATENCY_TARGET_MS,

# "required_bw_mbps":   REQUIRED_BW_MBPS,

# "util_steady_pct":    round(util_steady * 100, 2),

# "util_reload_pct":    round(util_reload * 100, 2),

# "buffer_kb_needed":   round(profile.buffer_kb_per_stream, 2),

# "stall_breakdown":    {k: round(v, 4) for k, v in stall_cpi(profile).items()},

# "hazard_classes":     sorted(HAZARD_TYPES & set().union(*HAZARD_EXPOSURE.values())),

# }

# def compare(a: ArchProfile, b: ArchProfile) -> dict:

# ra, rb = evaluate(a), evaluate(b)

# speedup = ra["exec_time_ms"] / rb["exec_time_ms"] if rb["exec_time_ms"] else float("inf")

# return {"A": ra, "B": rb, "speedup_B_over_A": round(speedup, 3)}

# # ======================================================================

# # 4. GUI  (CustomTkinter)

# # ======================================================================

# ctk.set_appearance_mode("dark")

# ctk.set_default_color_theme("blue")

# class ArchSimulatorApp(ctk.CTk):

# COLUMN_WIDTH = 46   # for aligned monospace text blocks

# def __init__(self):

# super().__init__()

# self.title("Inference Appliance - Architecture Comparator")

# self.geometry("980x680")

# self.minsize(860, 600)

# self._build_header()

# self._build_body()

# self._build_footer()

# self.refresh_results()

# # ------------------------------------------------------------------

# def _build_header(self):

# header = ctk.CTkFrame(self, fg_color="transparent")

# header.pack(fill="x", padx=20, pady=(18, 6))

# ctk.CTkLabel(

# header, text="Compact Multi-Stream Inference Appliance",

# font=ctk.CTkFont(size=22, weight="bold")

# ).pack(anchor="w")

# subtitle = (

# f"{STREAM_COUNT} streams x {PER_STREAM_MBPS:.0f} MB/s = "

# f"{REQUIRED_BW_MBPS:.0f} MB/s required | "

# f"{INSTRUCTIONS:,} instr/batch | target < {LATENCY_TARGET_MS:.1f} ms"

# )

# ctk.CTkLabel(header, text=subtitle, text_color="gray70").pack(anchor="w")

# # ------------------------------------------------------------------

# def _build_body(self):

# body = ctk.CTkFrame(self, fg_color="transparent")

# body.pack(fill="both", expand=True, padx=20, pady=6)

# body.grid_columnconfigure(0, weight=1)

# body.grid_columnconfigure(1, weight=1)

# body.grid_rowconfigure(0, weight=1)

# self.panel_a = self._make_result_panel(body, "Alternative A (In-Order)")

# self.panel_a.grid(row=0, column=0, sticky="nsew", padx=(0, 8))

# self.panel_b = self._make_result_panel(body, "Alternative B (Superscalar OoO)")

# self.panel_b.grid(row=0, column=1, sticky="nsew", padx=(8, 0))

# self.summary_box = ctk.CTkTextbox(self, height=150, font=("Consolas", 13))

# self.summary_box.pack(fill="x", padx=20, pady=(10, 10))

# def _make_result_panel(self, parent, title):

# frame = ctk.CTkFrame(parent, corner_radius=12)

# ctk.CTkLabel(frame, text=title, font=ctk.CTkFont(size=16, weight="bold")).pack(

# anchor="w", padx=16, pady=(14, 4))

# text = ctk.CTkTextbox(frame, font=("Consolas", 13), wrap="none")

# text.pack(fill="both", expand=True, padx=14, pady=(0, 14))

# frame.text_widget = text

# return frame

# # ------------------------------------------------------------------

# def _build_footer(self):

# footer = ctk.CTkFrame(self, fg_color="transparent")

# footer.pack(fill="x", padx=20, pady=(0, 18))

# ctk.CTkButton(footer, text="Recompute", command=self.refresh_results,

# width=140).pack(side="left")

# ctk.CTkLabel(footer, text="Adjust required bandwidth per stream (MB/s):").pack(

# side="left", padx=(20, 6))

# self.bw_slider_val = tk.DoubleVar(value=PER_STREAM_MBPS)

# slider = ctk.CTkSlider(footer, from_=4, to=32, number_of_steps=28,

# variable=self.bw_slider_val, command=self._on_slider)

# slider.pack(side="left", padx=6)

# self.bw_slider_label = ctk.CTkLabel(footer, text=f"{PER_STREAM_MBPS:.0f} MB/s")

# self.bw_slider_label.pack(side="left", padx=(6, 0))

# def _on_slider(self, _value):

# self.bw_slider_label.configure(text=f"{self.bw_slider_val.get():.0f} MB/s")

# # ------------------------------------------------------------------

# @staticmethod

# def _fmt_row(label: str, value: str, width: int = 34) -> str:

# return f"{label:<{width}}{value}\n"

# def _render_panel(self, frame, result: dict):

# w = frame.text_widget

# w.delete("1.0", "end")

# lines = []

# lines.append(self._fmt_row("Clock frequency", f"{result['clock_ghz']:.2f} GHz"))

# lines.append(self._fmt_row("CPI (with stalls)", f"{result['cpi']:.4f}"))

# lines.append(self._fmt_row("Execution time / batch", f"{result['exec_time_ms']:.4f} ms"))

# lines.append(self._fmt_row("Meets < 2.0 ms target?", "YES" if result["meets_latency"] else "NO"))

# lines.append("")

# lines.append(self._fmt_row("Required aggregate BW", f"{result['required_bw_mbps']:.1f} MB/s"))

# lines.append(self._fmt_row("Bandwidth util. (steady)", f"{result['util_steady_pct']:.2f} %"))

# lines.append(self._fmt_row("Bandwidth util. (reload)", f"{result['util_reload_pct']:.2f} %"))

# lines.append(self._fmt_row("Buffer needed / stream", f"{result['buffer_kb_needed']:.1f} KB"))

# lines.append("")

# lines.append("Stall CPI breakdown:")

# for cls, val in result["stall_breakdown"].items():

# lines.append(self._fmt_row(f"  {cls}", f"{val:.4f}", width=32))

# w.insert("1.0", "".join(lines))

# w.configure(state="disabled")

# def refresh_results(self):

# result = compare(ALTERNATIVE_A, ALTERNATIVE_B)

# self._render_panel(self.panel_a, result["A"])

# self._render_panel(self.panel_b, result["B"])

# speedup = result["speedup_B_over_A"]

# winner = "B (Superscalar OoO)" if speedup > 1 else "A (In-Order)"

# summary_lines = [

# self._fmt_row("Speed-up of B over A", f"{speedup:.3f}x", width=30),

# self._fmt_row("Recommended architecture", winner, width=30),

# self._fmt_row("Hazard classes modelled", ", ".join(sorted(HAZARD_TYPES)), width=30),

# ]

# self.summary_box.configure(state="normal")

# self.summary_box.delete("1.0", "end")

# self.summary_box.insert("1.0", "".join(summary_lines))

# self.summary_box.configure(state="disabled")

# # ======================================================================

# # 5. ENTRY POINT

# # ======================================================================

# if __name__ == "__main__":

# # Quick console sanity-check (also useful when customtkinter is absent)

# result = compare(ALTERNATIVE_A, ALTERNATIVE_B)

# print(f"{'Metric':<28}{'Alt A':>16}{'Alt B':>16}")

# print("-" * 60)

# print(f"{'CPI':<28}{result['A']['cpi']:>16.4f}{result['B']['cpi']:>16.4f}")

# print(f"{'Exec time (ms)':<28}{result['A']['exec_time_ms']:>16.4f}{result['B']['exec_time_ms']:>16.4f}")

# print(f"{'BW util steady (%)':<28}{result['A']['util_steady_pct']:>16.2f}{result['B']['util_steady_pct']:>16.2f}")

# print(f"Speed-up of B over A: {result['speedup_B_over_A']:.3f}x")

# print()

# app = ArchSimulatorApp()

# app.mainloop()

# OUTPUT